i-PRO Easy IP Setup Protocol — Technical Reference

Reverse-engineered from Wireshark captures of i-PRO EasyIP.exe. All multi-byte fields are big-endian unless noted.

1. Overview

The i-PRO Easy IP Setup protocol is a proprietary UDP broadcast protocol used by i-PRO (formerly Panasonic) cameras and network video recorders. It allows tools on the same Layer-2 subnet to:

- Discover all i-PRO devices and read their current network configuration

- Configure a device's IP address, subnet mask, gateway, DNS servers, HTTP port, and network mode (Static IP, DHCP, Auto-AutoIP, Auto-Advanced)

The protocol is connection-less and broadcast-based. No prior pairing or authentication is required — any host on the same broadcast domain can configure any device. The only device-targeting mechanism is the destination MAC address embedded in the packet payload.

2. Transport Layer

┌──────────────────┬─────────────────────────────┐

│    Parameter     │            Value            │

├──────────────────┼─────────────────────────────┤

│ Protocol         │ UDP                         │

├──────────────────┼─────────────────────────────┤

│ Source port      │ 10669 (0x29AD)              │

├──────────────────┼─────────────────────────────┤

│ Destination port │ 10670 (0x29AE)              │

├──────────────────┼─────────────────────────────┤

│ Destination IP   │ 255.255.255.255 (broadcast) │

├──────────────────┼─────────────────────────────┤

│ IP TTL           │ 128 (0x80)                  │

└──────────────────┴─────────────────────────────┘

Both discovery and configuration packets are sent to the broadcast address. Devices on the same subnet receive all packets and filter by the MAC address embedded in the payload.

3. General Packet Architecture

Every packet (discovery or configuration) shares this UDP payload layout:

Offset  Size  Field

──────  ────  ─────────────────────────────────────────────────────

0       2   Magic header          Always 0x00 0x01

2       2   Payload length        Length of the TLV block (0 if no TLV)

4       2   Command               See §4 and §5

6       6   Target device MAC     The camera/recorder MAC address

12       6   Sender MAC            Host (PC) MAC address

18       4   Sender IPv4           Host (PC) IPv4 address

22       2   Sequence number       0x0001 = config packet, 0x0002 = commit packet

24      24   Protocol constant     Fixed block (see §3.1)

48    0–175  TLV payload           Present only in config packets (see §5.2)

48+n    2   End marker            Always 0xFF 0xFF

50+n    2   Checksum              See §3.2

Packet sizes:

- Config packet: 48 + 175 (TLV) + 2 (end marker) + 2 (checksum) = 227 bytes UDP payload (UDP length field = 235)

- Commit packet: 48 + 0 + 2 + 2 = 52 bytes UDP payload (UDP length field = 60)

- Discovery search: approximately 96 bytes UDP payload

3.1 Protocol Constant Block (24 bytes)

Bytes 24–47 of every config and commit packet are always the same fixed sequence:

20 11 1E 11 23 1F 1E 19 13 00 00 02 02 00 00 00

00 00 00 00 00 00 00 00

The byte 0x02 at position 11 of this block (absolute payload offset 35) appears to be a device category flag that enables discovery of both cameras and recorders. Setting it to 0x01 limits responses to cameras only.

3.2 Checksum Algorithm

1. Assemble the full packet including the 0xFF 0xFF end marker

2. Sum every byte as an unsigned integer (no carry limit during accumulation)

3. Add 1 to the sum

4. Take the result modulo 65536 (mask to 16 bits)

5. Append as a 2-byte big-endian value

checksum = (sum(packet_bytes_including_end_marker) + 1) & 0xFFFF

Verified checksums from captures:

┌────────────────────────────┬──────────┐

│          Scenario          │ Checksum │

├────────────────────────────┼──────────┤

│ Static IP / Manual DNS     │ 0x28DA   │

├────────────────────────────┼──────────┤

│ DHCP / Auto DNS            │ 0x26A5   │

├────────────────────────────┼──────────┤

│ DHCP / Manual DNS          │ 0x2BDD   │

├────────────────────────────┼──────────┤

│ AutoIP / Auto DNS          │ 0x2BDF   │

├────────────────────────────┼──────────┤

│ AutoIP / Manual DNS        │ 0x29F1   │

├────────────────────────────┼──────────┤

│ Auto-Advanced / Auto DNS   │ 0x29F0   │

├────────────────────────────┼──────────┤

│ Auto-Advanced / Manual DNS │ 0x2938   │

├────────────────────────────┼──────────┤

│ Commit packet (all modes)  │ 0x09C4   │

└────────────────────────────┴──────────┘

The commit packet checksum is constant because the commit payload does not vary between modes — it contains no TLV data.

4. Discovery Protocol

4.1 Search Request (host → broadcast)

Command: 0x000D

The host broadcasts a search request. All i-PRO devices on the subnet reply with their current configuration.

UDP payload layout:

Offset  Size  Value / Description

──────  ────  ──────────────────────────────────────────────────────

0       4   00 01 00 2A           Header + payload length (0x2A = 42)

4       4   00 0D 00 00           Command 0x000D + flags

8       8   00 00 00 00 00 00     (padding / reserved)

16       6   [sender MAC]          Host MAC address

22       4   [sender IPv4]         Host IP address

26       2   00 00                 Padding

28      11   00 00 20 11 1E 11     Protocol signature block

23 1F 1E 19 13

39      15   00 00 02 01 00 00     Capability / category block

00 00 00 00 00 00

00 00 00              (0x02 at offset 41 enables recorder discovery)

54       2   FF F0                 Category flags

56      42   00 26 00 20 00 21     Requested TLV tags list (21 tag IDs × 2 bytes each,

00 22 00 23 00 25     terminated by FF FF)

00 28 00 40 00 41

00 42 00 44 00 A5

00 A6 00 A7 00 A8

00 AD 00 B3 00 B4

00 B7 00 B8 FF FF

98       2   11 70                 Trailer (required for recorder discovery)

The list of requested tag IDs tells the device which TLV fields to include in its response.

4.2 Search Response (device → host unicast)

Devices reply with a unicast UDP packet to the host's source port. The response uses the same general header structure

but carries device information in TLV format starting at payload offset 0x30 (48 decimal).

Response TLV tags:

┌───────────┬────────┬────────────────────────────────────────────────────────────────┐

│ Tag (hex) │ Length │                          Description                           │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x0000    │ 1      │ Network mode (see §4.3)                                        │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x0020    │ 4      │ IPv4 address                                                   │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x0021    │ 4      │ Subnet mask                                                    │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x0022    │ 4      │ Default gateway                                                │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x0025    │ 2      │ HTTP port                                                      │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00A6    │ 1      │ Device/DNS type code                                           │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00A7    │ var    │ Device name (null-terminated UTF-8)                            │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00A8    │ var    │ Model name (null-terminated UTF-8)                             │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00A9    │ var    │ Firmware version (null-terminated UTF-8)                       │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00C0    │ 2      │ Channel count (recorders only — use to identify recorder type) │

├───────────┼────────┼────────────────────────────────────────────────────────────────┤

│ 0x00D1    │ var    │ Serial number (null-terminated UTF-8)                          │

└───────────┴────────┴────────────────────────────────────────────────────────────────┘

Each TLV entry is: [tag: 2 bytes] [length: 2 bytes] [value: length bytes]. The block ends with 0xFF 0xFF.

4.3 Network Mode Values in Responses

The network mode byte (tag 0x0000) in response packets uses this encoding:

┌───────┬─────────────────┐

│ Value │      Mode       │

├───────┼─────────────────┤

│ 0x00  │ DHCP            │

├───────┼─────────────────┤

│ 0x02  │ Static IP       │

├───────┼─────────────────┤

│ 0x04  │ Auto (AutoIP)   │

├───────┼─────────────────┤

│ 0x05  │ Auto (Advanced) │

└───────┴─────────────────┘

Note: these values differ from the mode bytes used in config packets (§5.3). The device encodes mode differently when reporting vs. accepting configuration.

5. Configuration Protocol

Changing a device's network settings requires two distinct packet types sent in sequence: a Config packet (command 0x0002) and a Commit packet (command 0x0004). The device does not apply settings until it receives the commit.

5.1 Transaction Sequence and Timing

Host                                   Device

│                                        │

│── Config packet (×3, 100 ms apart)  ──▶│  Device stores pending config

│                                        │

│  [wait 200 ms]                         │

│                                        │

│── Commit packet (×3, 100 ms apart)  ──▶│  Device applies config, may reboot

│                                        │

│◀─── Optional response ──────────────── │  Device may or may not reply

Sending each packet three times compensates for UDP packet loss on busy broadcast segments. The device applies the most recently received valid config when it receives the commit.

5.2 Config Packet — TLV Payload (175 bytes / 0xAF)

The TLV payload begins at byte offset 48 of the UDP payload. It is always exactly 175 bytes (0xAF).

5.2.1 Pre-TLV Prefix (10 bytes)

The payload opens with a 10-byte prefix that is not in standard TLV format:

Byte  Value   Description

────  ─────   ────────────────────────────────────────────────

0   0x00    Constant

1   0x00    Constant

2   0x00    Constant

3   0x01    Constant

4   [MODE]  Network mode byte (see §5.3)

5   0x00    Constant

6   0x01    Constant

7   0x00    Constant

8   0x01    Constant

9   0x00    Constant

Byte 4 is the only variable byte in this prefix. All other bytes are constant across all observed captures.

5.2.2 TLV Entries

Following the 10-byte prefix, the remaining 165 bytes consist of standard TLV entries, each encoded as [tag: 2 bytes big-endian] [length: 2 bytes big-endian] [value: length bytes]:

┌────────┬────────┬──────────────────────────┬──────────────────────────────────────────────────────┐

│  Tag   │ Length │       Description        │                        Notes                         │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0020 │ 4      │ IPv4 address             │ Dotted-quad, big-endian                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0021 │ 4      │ Subnet mask              │ Dotted-quad, big-endian                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0022 │ 4      │ Default gateway          │ Dotted-quad, big-endian                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0023 │ 8      │ DNS servers              │ Primary (4 bytes) + Secondary (4 bytes)              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0025 │ 2      │ HTTP port                │ Big-endian uint16 — must match device's current port │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0040 │ 16     │ IPv6 link-local address  │ EUI-64 derived from target MAC (see §5.5)            │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0041 │ 16     │ IPv6 global address      │ All zeros                                            │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0042 │ 32     │ Unknown IPv6 block       │ All zeros                                            │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x0044 │ 2      │ HTTPS port               │ Always 443 (0x01BB) in all observed captures         │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x00A0 │ 4      │ IPv4 address (mirror)    │ Identical to tag 0x0020                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x00A1 │ 4      │ Subnet mask (mirror)     │ Identical to tag 0x0021                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x00A2 │ 4      │ Default gateway (mirror) │ Identical to tag 0x0022                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x00A3 │ 8      │ DNS servers (mirror)     │ Identical to tag 0x0023                              │

├────────┼────────┼──────────────────────────┼──────────────────────────────────────────────────────┤

│ 0x00A6 │ 1      │ DNS mode flag            │ 0x90 = Auto DNS, 0x92 = Manual DNS                   │

└────────┴────────┴──────────────────────────┴──────────────────────────────────────────────────────┘

The TLV block terminates at 0xFF 0xFF followed by the 2-byte checksum. There is no explicit end marker within the TLV itself.

TLV byte accounting:

┌────────────────────────────────┬───────┐

│            Section             │ Bytes │

├────────────────────────────────┼───────┤

│ Pre-TLV prefix                 │ 10    │

├────────────────────────────────┼───────┤

│ Tag 0x0020 (4+4 overhead+data) │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x0021                     │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x0022                     │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x0023 (4+8)               │ 12    │

├────────────────────────────────┼───────┤

│ Tag 0x0025 (4+2)               │ 6     │

├────────────────────────────────┼───────┤

│ Tag 0x0040 (4+16)              │ 20    │

├────────────────────────────────┼───────┤

│ Tag 0x0041 (4+16)              │ 20    │

├────────────────────────────────┼───────┤

│ Tag 0x0042 (4+32)              │ 36    │

├────────────────────────────────┼───────┤

│ Tag 0x0044 (4+2)               │ 6     │

├────────────────────────────────┼───────┤

│ Tag 0x00A0                     │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x00A1                     │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x00A2                     │ 8     │

├────────────────────────────────┼───────┤

│ Tag 0x00A3                     │ 12    │

├────────────────────────────────┼───────┤

│ Tag 0x00A6 (4+1)               │ 5     │

├────────────────────────────────┼───────┤

│ Total                          │ 175   │

└────────────────────────────────┴───────┘

5.3 Network Mode Byte (Pre-TLV prefix, byte 4)

┌───────┬─────────────────┬────────────────────────────────────────────────────────────────────────┐

│ Value │      Mode       │                                 Notes                                  │

├───────┼─────────────────┼────────────────────────────────────────────────────────────────────────┤

│ 0x00  │ DHCP            │ Device requests IP from DHCP server                                    │

├───────┼─────────────────┼────────────────────────────────────────────────────────────────────────┤

│ 0x03  │ Static IP       │ Device uses the IP in tags 0x20/0xA0 permanently                       │

├───────┼─────────────────┼────────────────────────────────────────────────────────────────────────┤

│ 0x04  │ Auto (AutoIP)   │ DHCP with link-local (169.254.x.x) fallback if no DHCP server responds │

├───────┼─────────────────┼────────────────────────────────────────────────────────────────────────┤

│ 0x05  │ Auto (Advanced) │ DHCP + AutoIP + additional auto-negotiation                            │

└───────┴─────────────────┴────────────────────────────────────────────────────────────────────────┘

In all modes, the IPv4 address, subnet, and gateway TLV fields must be populated with valid values. For DHCP and auto modes, the device uses these as a fallback or ignores them after obtaining a DHCP lease.

Important: The mode byte values in config packets (0x03 for Static) differ from the mode values returned by devices in discovery responses (0x02 for Static). The two encodings are not the same.

5.4 DNS Mode Flag (Tag 0x00A6)

Tag 0x00A6 controls whether the device uses the DNS servers embedded in tags 0x0023/0x00A3, or obtains DNS automatically:

┌───────┬──────────────────────────────────────────────────────────────────┐

│ Value │                             DNS Mode                             │

├───────┼──────────────────────────────────────────────────────────────────┤

│ 0x90  │ Auto — device obtains DNS from DHCP or uses network-provided DNS │

├───────┼──────────────────────────────────────────────────────────────────┤

│ 0x92  │ Manual — device uses the addresses in tags 0x0023 / 0x00A3       │

└───────┴──────────────────────────────────────────────────────────────────┘

When DNS mode is Auto, the DNS server bytes in tags 0x0023 and 0x00A3 serve as a placeholder only and appear to be   ignored by the device. The original EasyIP.exe populates these with 8.8.8.8 / 8.8.4.4 when DNS Auto is selected with DHCP, or with the device's current DNS when DNS Auto is selected with AutoIP/Auto-Advanced modes.

The correlation across all captured scenarios:

┌─────────────────┬──────────┬───────────┬──────┐

│  Network Mode   │ DNS Mode │ Mode byte │ 0xA6 │

├─────────────────┼──────────┼───────────┼──────┤

│ Static IP       │ Manual   │ 0x03      │ 0x92 │

├─────────────────┼──────────┼───────────┼──────┤

│ DHCP            │ Auto     │ 0x00      │ 0x90 │

├─────────────────┼──────────┼───────────┼──────┤

│ DHCP            │ Manual   │ 0x00      │ 0x92 │

├─────────────────┼──────────┼───────────┼──────┤

│ Auto (AutoIP)   │ Auto     │ 0x04      │ 0x90 │

├─────────────────┼──────────┼───────────┼──────┤

│ Auto (AutoIP)   │ Manual   │ 0x04      │ 0x92 │

├─────────────────┼──────────┼───────────┼──────┤

│ Auto (Advanced) │ Auto     │ 0x05      │ 0x90 │

├─────────────────┼──────────┼───────────┼──────┤

│ Auto (Advanced) │ Manual   │ 0x05      │ 0x92 │

└─────────────────┴──────────┴───────────┴──────┘

5.5 IPv6 Link-Local Address Derivation (EUI-64)

Tag 0x0040 carries the IPv6 link-local address derived from the target device's MAC address using the standard EUI-64

procedure:

1. Split the 6-byte MAC into its two 3-byte halves: [OUI] and [NIC]

2. Insert 0xFF 0xFE between them to form an 8-byte interface identifier

3. Flip bit 6 (the "Universal/Local" bit) of the first byte: first_byte ^= 0x02

4. Prepend the link-local prefix FE80:: (10 bytes: FE 80 00 00 00 00 00 00)

Example: MAC D4:2D:C5:2A:8D:13

OUI = D4 2D C5   →   D4 ^ 02 = D6

NIC = 2A 8D 13

EUI-64 interface ID = D6 2D C5 FF FE 2A 8D 13

IPv6 link-local = FE80::D62D:C5FF:FE2A:8D13

Bytes = FE 80 00 00 00 00 00 00 D6 2D C5 FF FE 2A 8D 13

This matches the 0x0040 value observed in all captures.

5.6 HTTP Port Constraint

The HTTP port value in tag 0x0025 must match the device's current HTTP port. If it does not match, the device silently discards the config packet without any error response. This means:

1. Before configuring a device, the current port must be known (obtained from discovery)

2. The port value in the config packet sets the new HTTP port, not just validates it — but since the device rejects packets with a non-matching port, the port value effectively cannot be changed through this protocol alone

6. Commit Packet

Command: 0x0004

Offset  Size  Value

──────  ────  ──────────────────────────────────────────

0       2   00 01   (magic header)

2       2   00 00   (payload length = 0, no TLV)

4       2   00 04   (command: commit)

6       6   [target device MAC]

12       6   [sender MAC]

18       4   [sender IPv4]

22       2   00 02   (sequence number = 2)

24      24   [protocol constant block]

48       2   FF FF   (end marker)

50       2   [checksum]

The commit packet is identical for all network modes and DNS configurations — it carries no mode information. The checksum is always 0x09C4 when using the same sender and target MAC/IP addresses.

---

7. Annotated Hex Dump — Static IP / Manual DNS

Sender: A0:29:19:3E:AB:91 at 192.168.1.121 configuring camera D4:2D:C5:2A:8D:13 to 192.168.1.54 / 255.255.255.0

gateway 192.168.1.1, DNS 192.168.1.250 + 8.8.4.4, port 443.

Config packet (config sent ×3):

Ethernet + IP + UDP headers (42 bytes):

FF FF FF FF FF FF           Dst MAC: broadcast

A0 29 19 3E AB 91           Src MAC: sender

08 00                       EtherType: IPv4

45 00 00 FF ...             IP header (20 bytes)

29 AD 29 AE 00 EB C3 BB     UDP: sport=10669 dport=10670 len=235

UDP Payload (227 bytes):

00 01                       Magic header

00 AF                       TLV length: 175

00 02                       Command: Config

D4 2D C5 2A 8D 13           Target MAC (camera)

A0 29 19 3E AB 91           Sender MAC (host)

C0 A8 01 79                 Sender IP: 192.168.1.121

00 01                       Sequence: 1 (config)

20 11 1E 11 23 1F 1E 19     Protocol constant (24 bytes)

13 00 00 02 02 00 00 00

00 00 00 00 00 00 00 00

[TLV Payload — 175 bytes]

00 00 00 01 03 00 01 00 01 00   Pre-TLV prefix (mode byte = 0x03 = Static IP)

00 20 00 04  C0 A8 01 36        Tag 0x0020: IP = 192.168.1.54

00 21 00 04  FF FF FF 00        Tag 0x0021: Subnet = 255.255.255.0

00 22 00 04  C0 A8 01 01        Tag 0x0022: Gateway = 192.168.1.1

00 23 00 08  C0 A8 01 FA        Tag 0x0023: DNS primary = 192.168.1.250

08 08 04 04                    DNS secondary = 8.8.4.4

00 25 00 02  01 BB              Tag 0x0025: HTTP port = 443

00 40 00 10  FE 80 00 00 00 00  Tag 0x0040: IPv6 link-local (EUI-64)

00 00 D6 2D C5 FF

FE 2A 8D 13

00 41 00 10  00×16              Tag 0x0041: IPv6 global (zeros)

00 42 00 20  00×32              Tag 0x0042: Unknown (zeros)

00 44 00 02  01 BB              Tag 0x0044: HTTPS port = 443

00 A0 00 04  C0 A8 01 36        Tag 0x00A0: IP mirror = 192.168.1.54

00 A1 00 04  FF FF FF 00        Tag 0x00A1: Subnet mirror

00 A2 00 04  C0 A8 01 01        Tag 0x00A2: Gateway mirror

00 A3 00 08  C0 A8 01 FA        Tag 0x00A3: DNS mirror

08 08 04 04

00 A6 00 01  92                 Tag 0x00A6: DNS mode = 0x92 (Manual)

FF FF                           End marker

28 DA                           Checksum

Commit packet (sent ×3, 200 ms after last config):

UDP Payload (52 bytes):

00 01                       Magic header

00 00                       TLV length: 0 (no TLV)

00 04                       Command: Commit

D4 2D C5 2A 8D 13           Target MAC (camera)

A0 29 19 3E AB 91           Sender MAC (host)

C0 A8 01 79                 Sender IP

00 02                       Sequence: 2 (commit)

20 11 1E 11 23 1F 1E 19     Protocol constant (24 bytes)

13 00 00 02 02 00 00 00

00 00 00 00 00 00 00 00

FF FF                       End marker

09 C4                       Checksum

8. Mode Comparison — What Changes Per Mode

The only bytes that vary between different configuration scenarios are:

┌────────────────────────────────────────────────┬──────────────┬────────────────────────────────────────┐

│                    Location                    │     Size     │        What it controls                │ ├────────────────────────────────────────────────┼──────────────┼────────────────────────────────────────┤

│ TLV prefix byte 4 (payload offset 52)          │ 1 byte       │ Network mode                           │

│                                                │              │ (Static/DHCP/AutoIP/Advanced)          │

├────────────────────────────────────────────────┼──────────────┼────────────────────────────────────────┤

│ Tag 0x0023 value (8 bytes)                     │ 8 bytes      │ DNS server addresses                   │ ├────────────────────────────────────────────────┼──────────────┼────────────────────────────────────────┤

│ Tag 0x00A3 value (8 bytes)                     │ 8 bytes      │ DNS server addresses (mirror)          │

├────────────────────────────────────────────────┼──────────────┼────────────────────────────────────────┤

│ Tag 0x00A6 value (1 byte)                      │ 1 byte       │ DNS mode (Auto vs Manual)              │ ├────────────────────────────────────────────────┼──────────────┼────────────────────────────────────────┤

│ Tags 0x0020/0x0021/0x0022/0x00A0/0x00A1/0x00A2 │ 4 bytes each │ IP / subnet / gateway                  │

└────────────────────────────────────────────────┴──────────────┴────────────────────────────────────────┘

Everything else — the 24-byte protocol constant, all TLV tag IDs and lengths, the 0x0040–0x0044 IPv6 block — is identical across all modes.

9. Implementation Notes

Port matching requirement. The device silently ignores config packets where tag 0x0025 does not match its current HTTP port. Always discover the device first to obtain the live port value.

Duplicate packet transmission. The original tool sends each packet exactly three times, 100 ms apart. This is pure loss compensation — UDP on a broadcast segment can collide. The device processes only the first valid packet;   duplicates are harmless.

No authentication. Any host on the broadcast segment can configure any device by knowing (or guessing) its MAC address. MAC addresses are trivially discoverable via the discovery protocol.

No response to commit. Some devices respond to the config packet but not the commit. Others reboot immediately on commit and send nothing. Absence of a response is not an error.

Recorder detection. Device type cannot be reliably determined from tag 0x00A6 in discovery responses (its value varies with DNS mode, not device type). Recorders are identified by either the presence of tag 0x00C0 (channel count) in the response, or a model name beginning with NX or WJ.

IPv6 tags are mandatory. Even though tags 0x0041 and 0x0042 carry only zeros, and tag 0x0040 carries a computed value that is never acted on in DHCP/auto modes, all three must be present. Omitting them changes the TLV length from 175 bytes, which causes the device to reject the packet.